# Interceptor AI Module - Project Report

This document outlines the development progression, architectural changes, and problem resolutions since the inception of the Interceptor AI project, with a specific focus on optimizations for the main AI model and edge deployment platforms like Radxa.

# 1. Project Overview & Initial Implementation

The project began as an FPV Interceptor AI designed to provide visual lock and follow capabilities for INAV FPV drones. The initial architecture was established with the following core components:

Hybrid Tracking System: Integration of Ultralytics YOLO with OpenCV (CSRT/KCF) to provide stable frame-to-frame tracking while minimizing compute overhead.

Flight Control Link: MSP-based direct RC overrides (yaw and pitch) to steer the drone in ANGLE mode.

Training Pipeline: Custom dataset scripts (drone_missile) to allow fine-tuning on specific targets.

# 2. Architectural Evolution and GUI Development

As the project matured, a major refactoring effort transformed the codebase from a set of scripts into a robust, modular application.

Modular Core: Introduction of Plugin Managers, Config Managers, and an AI Model Manager to allow dynamic loading of trackers and flight controllers.

Advanced GUI: Transition to a comprehensive dashboard featuring telemetry widgets, PID tuning panels, video processing widgets, and an overarching Arjuna Shell.

Database Integration: Implementation of a local Target Store and Profile database to persistently manage tracking subjects and their parameters.

Advanced Tracking: Added Kalman filters and motion predictors to anticipate target trajectories during frame drops or occlusions.

# 3. Solutions for the Main Model

Several challenges regarding the primary detection model (YOLO) were addressed to improve accuracy and efficiency:

Model Selection & Inference: Standardized on YOLOv8 Nano (yolov8n.pt) as the baseline for high-speed inference, balancing accuracy with computational limits.

Hybrid Reacquisition: Solved the issue of the model losing fast-moving targets by implementing a logic where the OpenCV tracker handles intermediate frames, while YOLO reacquires the target every N frames (YOLO_EVERY_N).

Open-Vocabulary Tracking: Introduced integration for YOLO-World, allowing the system to track objects based on text prompts (e.g., "drone", "UAV") without requiring retraining.

Hardware Acceleration: Seamless fallback mechanisms between CUDA (for high-end ground stations) and CPU execution.

# 4. Optimizations for Edge Deployment (Radxa & SBCs)

Deploying computer vision models on Single Board Computers (SBCs) like Radxa presents severe compute constraints. The following optimizations solved deployment blockers:

Throttled AI Inference: By configuring YOLO_EVERY_N, the heavy neural network runs only at a fraction of the camera frame rate. The lightweight KCF/CSRT tracker handles the rest, keeping CPU usage low on the Radxa.

Resolution Scaling: Implemented normalized error calculations allowing the vision pipeline to run at lower resolutions (e.g., 640x480 or 320x240) while still providing accurate PID feedback to the flight controller.

Executable Build Pipeline: Created build_exe.py and build_exe.bat to compile the entire Python environment into a standalone executable. This eliminates complex dependency management (like PyTorch and OpenCV installation) on the target edge device.

Headless/Safe Mode Configurations: Added safe-mode flags and diagnostic scripts to ensure the module can recover gracefully if GUI components fail in headless edge environments.

# 5. Flight Control & PID Enhancements

To ensure the drone accurately intercepts the target, the control loop was heavily refined:

Distance Estimation: Added real-time distance estimation to scale the PID responsiveness based on how far the target is.

Lead Calculation: Solved latency issues by predicting where the target will be based on its image-plane velocity.

Safety & Failsafes: Added Failsafe Managers and Arm checks to prevent runaway conditions if telemetry or visual lock is lost.